AEROSTACK2
Dynamic Multi-Drone Area Survey
and Task Reallocation

ROS 2 Humble and Gazebo Based Multi-Drone Autonomous Survey
with Dynamic Work Transfer and Mission Recovery

Source file: area_survey.py
Complete source captured from the project terminal and included for mentor reference.

#!/usr/bin/env python3

import time

import threading

import rclpy

from as2_python_api.drone_interface import DroneInterface

# CONFIGURATION

TAKEOFF_HEIGHT = 1.0

TAKEOFF_SPEED = 0.7

LAND_SPEED = 0.4

MISSION_SPEED = 1.0

STABILIZATION_TIME = 3.0

# Simulated delayed-drone condition

SIMULATED_DELAY = 8.0

DELAY_DRONE = "drone0"

# Zero-based index:

# 0 = waypoint 1

# 1 = waypoint 2

# 2 = waypoint 3

# 3 = waypoint 4

DELAY_WAYPOINT_INDEX = 2

# Time to wait for a naturally idle helper.

HELPER_WAIT_TIMEOUT = 5.0

# Maximum time allowed for a stopped helper thread to exit.

PREEMPT_STOP_TIMEOUT = 8.0

# Maximum time allowed for one asynchronous GoTo

GOTO_TIMEOUT = 60.0

# SURVEY AREAS

AREA_A = [

(-2.5, -2.0, 1.0),

(-2.5,  2.0, 1.0),

(-1.0,  2.0, 1.0),

(-1.0, -2.0, 1.0),

]

AREA_B = [

(-0.5, -2.0, 1.0),

(-0.5,  2.0, 1.0),

( 0.5,  2.0, 1.0),

( 0.5, -2.0, 1.0),

]

AREA_C = [

(1.0, -2.0, 1.0),

(1.0,  2.0, 1.0),

(2.5,  2.0, 1.0),

(2.5, -2.0, 1.0),

]

AREAS = {

"A": AREA_A,

"B": AREA_B,

"C": AREA_C,

}

INITIAL_ASSIGNMENT = {

"drone0": "A",

"drone1": "B",

"drone2": "C",

}

# DRONE CLASS

class SurveyDrone(DroneInterface):

def __init__(self, namespace):

super().__init__(

namespace,

verbose=True,

use_sim_time=True

)

self.name = namespace

# Mission state

self.mission_state = "READY"

# Original assigned area

self.original_area = None

self.original_waypoints = []

# Waypoint progress for original area

self.completed_indices = set()

self.current_index = None

# Preemption control

self.preempt_requested = False

# Thread reference

self.thread = None

# Drone0 delay detection

self.delay_event = threading.Event()

self.reassignment_event = threading.Event()

# Thread safety

self.lock = threading.Lock()

# STATE

def set_state(self, state):

with self.lock:

self.mission_state = state

def get_state(self):

with self.lock:

return self.mission_state

def get_completed_indices(self):

with self.lock:

return set(self.completed_indices)

def get_remaining_indices(self):

with self.lock:

completed = set(self.completed_indices)

return [

i

for i in range(len(self.original_waypoints))

if i not in completed

]

# GO TO

def execute_go_to(self, point):

x, y, z = point

print(

f"{self.name}: GO_TO "

f"({x:.2f}, {y:.2f}, {z:.2f})"

)

try:

# Do not start a new GoTo while another one is active.

if self.go_to.is_running():

print(

f"{self.name}: ERROR - GoTo handler is still running"

)

return False

# ASYNCHRONOUS GOTO

accepted = self.go_to(

x,

y,

z,

MISSION_SPEED,

0,

None,

"earth",

False

)

if not accepted:

print(

f"{self.name}: GoTo goal was not accepted"

)

return False

start_time = time.monotonic()

# Wait for the asynchronous behavior to finish.

while self.go_to.is_running():

# Safety timeout.

if (

time.monotonic() - start_time

> GOTO_TIMEOUT

):

print(

f"{self.name}: GoTo timeout after "

f"{GOTO_TIMEOUT:.1f}s"

)

try:

self.go_to.stop()

except Exception:

pass

return False

time.sleep(0.1)

# If the behavior ended because this drone was

# preempted, this waypoint was NOT completed.

with self.lock:

was_preempted = self.preempt_requested

if was_preempted:

return False

print(

f"{self.name}: GoTo behavior finished"

)

return True

except Exception as exc:

print(

f"{self.name}: GoTo exception: {exc}"

)

return False

# STOP CURRENT BEHAVIOR

def stop_current_behavior(self):

try:

if self.go_to.is_running():

print(

f"{self.name}: stopping current GoTo behavior..."

)

result = self.go_to.stop()

print(

f"{self.name}: GoTo stop result = {result}"

)

return bool(result)

else:

print(

f"{self.name}: no active GoTo behavior"

)

return True

except Exception as exc:

print(

f"{self.name}: stop exception: {exc}"

)

return False

# INITIAL SURVEY THREAD

def run_initial_survey(

self,

manager,

area_name,

waypoints,

delay_enabled=False

):

self.original_area = area_name

self.original_waypoints = list(waypoints)

self.set_state("SURVEYING")

print()

print("=" * 60)

print(

f"{self.name}: STARTING AREA {area_name}"

)

print("=" * 60)

for i, point in enumerate(waypoints):

# Check preemption before starting next waypoint

with self.lock:

if self.preempt_requested:

self.mission_state = "PREEMPTED"

print(

f"{self.name}: PREEMPTED "

f"before waypoint {i + 1}"

)

return

self.current_index = i

# Simulated Drone0 delay

if delay_enabled and i == DELAY_WAYPOINT_INDEX:

self.set_state("DELAYED")

print()

print("=" * 60)

print("SIMULATED DELAY")

print("=" * 60)

print(

f"{self.name}: DELAY detected before "

f"waypoint {i + 1}"

)

print(

f"Simulated delay condition = "

f"{SIMULATED_DELAY:.1f} seconds"

)

print(

f"{self.name}: waiting for "

f"task reassignment..."

)

# Tell mission manager immediately.

self.delay_event.set()

# Wait until mission manager transfers

# the remaining task.

while not self.reassignment_event.wait(0.2):

if self.get_state() == "FAILED":

return

self.set_state("TRANSFERRED")

print(

f"{self.name}: remaining work "

f"transferred to helper"

)

return

# Execute waypoint

success = self.execute_go_to(point)

with self.lock:

was_preempted = self.preempt_requested

if success:

self.completed_indices.add(i)

manager.mark_complete(area_name, i)

if success:

print(

f"{self.name}: waypoint {i + 1} "

f"COMPLETED"

)

else:

if was_preempted:

self.set_state("PREEMPTED")

print(

f"{self.name}: waypoint {i + 1} "

f"interrupted by preemption"

)

return

else:

self.set_state("FAILED")

print(

f"{self.name}: waypoint {i + 1} FAILED"

)

return

# If stop happened immediately after GoTo success

with self.lock:

if self.preempt_requested:

self.mission_state = "PREEMPTED"

print(

f"{self.name}: preemption detected "

f"after waypoint {i + 1}"

)

return

# Entire original area completed

self.set_state("AVAILABLE")

print()

print(

f"{self.name}: AREA {area_name} "

f"COMPLETED"

)

# MISSION MANAGER

class AreaSurveyManager:

def __init__(self):

self.drones = {}

self.completed_work = set()

self.progress_lock = threading.Lock()

self.reallocation_done = False

self.reallocation_helper = None

self.reallocation_record = None

# MARK GLOBAL WAYPOINT COMPLETE

def mark_complete(self, area_name, index):

with self.progress_lock:

self.completed_work.add(

(area_name, index)

)

# CHECK AREA PROGRESS

def area_progress(self, area_name):

with self.progress_lock:

return sum(

1

for area, _ in self.completed_work

if area == area_name

)

# EXECUTE A TASK ON A DRONE

def execute_reassigned_task(

self,

drone,

area_name,

waypoints,

indices,

task_description

):

print()

print("=" * 60)

print(task_description)

print("=" * 60)

if not indices:

print(

f"{drone.name}: no remaining waypoints"

)

return True

drone.set_state("REASSIGNED")

for index in indices:

point = waypoints[index]

print()

print(

f"{drone.name}: assigned "

f"Area {area_name} waypoint "

f"{index + 1}/{len(waypoints)}"

)

success = drone.execute_go_to(point)

if not success:

print()

print(

f"{drone.name}: FAILED reassigned "

f"waypoint {index + 1}"

)

drone.set_state("FAILED")

return False

self.mark_complete(

area_name,

index

)

print(

f"{drone.name}: Area {area_name} "

f"waypoint {index + 1} COMPLETED"

)

return True

# FIND NATURALLY AVAILABLE HELPER

def find_naturally_available_helper(self):

helpers = [

self.drones["drone1"],

self.drones["drone2"],

]

for helper in helpers:

if helper.get_state() == "AVAILABLE":

return helper

return None

# SELECT BUSY HELPER FOR PREEMPTION

def select_busy_helper(self):

candidates = []

for name in ["drone2", "drone1"]:

drone = self.drones[name]

state = drone.get_state()

if state == "SURVEYING":

candidates.append(drone)

if not candidates:

return None

# Prefer the drone that has already completed

# more of its current survey.

candidates.sort(

key=lambda d: len(d.get_completed_indices()),

reverse=True

)

return candidates[0]

# PREEMPT HELPER

def preempt_helper(self, helper):

print()

print("=" * 60)

print("PREEMPTING BUSY HELPER")

print("=" * 60)

print(

f"Selected helper: {helper.name}"

)

print(

f"Current state: {helper.get_state()}"

)

print(

f"Original area: "

f"{helper.original_area}"

)

print(

f"Completed original waypoints: "

f"{sorted(helper.get_completed_indices())}"

)

# Tell worker thread not to start another

# waypoint after the current GoTo ends.

with helper.lock:

helper.preempt_requested = True

# Stop current GoTo.

stop_result = helper.stop_current_behavior()

print(

f"{helper.name}: stop request = "

f"{stop_result}"

)

# Wait for the old survey thread to exit.

if helper.thread is not None:

helper.thread.join(

timeout=PREEMPT_STOP_TIMEOUT

)

if (

helper.thread is not None

and helper.thread.is_alive()

):

print()

print(

f"ERROR: {helper.name} survey thread "

f"did not stop safely."

)

return None

remaining = helper.get_remaining_indices()

print()

print(

f"{helper.name}: original survey "

f"successfully preempted"

)

print(

f"{helper.name}: unfinished original "

f"waypoints = "

f"{[i + 1 for i in remaining]}"

)

helper.set_state("PREEMPTED")

# The old survey worker has completely finished.

# Clear the flag before giving this drone a new task.

with helper.lock:

helper.preempt_requested = False

# Give the action server a short time to settle.

time.sleep(0.75)

return remaining

# WAIT FOR HELPER

def wait_for_helper(self):

print()

print("=" * 60)

print("SEARCHING FOR HELPER DRONE")

print("=" * 60)

start = time.monotonic()

while (

time.monotonic() - start

< HELPER_WAIT_TIMEOUT

):

helper = (

self.find_naturally_available_helper()

)

if helper is not None:

print(

f"Available helper found: "

f"{helper.name}"

)

return helper

elapsed = (

time.monotonic() - start

)

remaining = max(

0.0,

HELPER_WAIT_TIMEOUT - elapsed

)

print(

"No idle helper yet. "

f"Waiting {remaining:.1f}s..."

)

time.sleep(1.0)

print()

print(

"No helper became naturally available."

)

print(

"Switching to PREEMPTIVE REALLOCATION."

)

return self.select_busy_helper()

# DYNAMIC REALLOCATION

def perform_reallocation(self):

drone0 = self.drones["drone0"]

remaining_a = drone0.get_remaining_indices()

print()

print("=" * 60)

print("DYNAMIC TASK REALLOCATION")

print("=" * 60)

print(

"Drone0 = DELAYED"

)

print(

f"Area A completed: "

f"{len(drone0.get_completed_indices())}/"

f"{len(AREA_A)}"

)

print(

"Drone0 remaining waypoints:"

)

for index in remaining_a:

point = AREA_A[index]

print(

f"  A{index + 1}: "

f"({point[0]:.2f}, "

f"{point[1]:.2f}, "

f"{point[2]:.2f})"

)

if not remaining_a:

print(

"Drone0 has no remaining work."

)

drone0.reassignment_event.set()

return True

# Find helper

helper = self.wait_for_helper()

if helper is None:

print()

print(

"ERROR: No helper drone available "

"for reallocation."

)

return False

helper_remaining = []

if helper.get_state() == "SURVEYING":

helper_remaining = (

self.preempt_helper(helper)

)

if helper_remaining is None:

return False

elif helper.get_state() == "AVAILABLE":

helper_remaining = (

helper.get_remaining_indices()

)

else:

print(

f"ERROR: Helper {helper.name} "

f"is in state {helper.get_state()}"

)

return False

self.reallocation_helper = helper.name

self.reallocation_record = {

"source_drone": "drone0",

"helper_drone": helper.name,

"area": "A",

"waypoints": list(remaining_a),

}

print()

print("=" * 60)

print("TASK TRANSFER")

print("=" * 60)

print(

f"Drone0 -> {helper.name}"

)

print(

f"Transferred Area A waypoints: "

f"{[i + 1 for i in remaining_a]}"

)

success = self.execute_reassigned_task(

helper,

"A",

AREA_A,

remaining_a,

f"{helper.name}: EXECUTING DRONE0 REMAINING WORK"

)

if not success:

print(

"ERROR: Reassigned Area A task failed."

)

return False

print()

print("=" * 60)

print("DRONE0 WORK COMPLETED BY HELPER")

print("=" * 60)

print(

f"Area A progress: "

f"{self.area_progress('A')}/4"

)

# Tell Drone0 its remaining task has been transferred.

drone0.reassignment_event.set()

# Resume helper's original area.

if helper_remaining:

print()

print("=" * 60)

print(

f"RESUMING {helper.name} ORIGINAL SURVEY"

)

print("=" * 60)

print(

f"Original area: "

f"{helper.original_area}"

)

print(

"Remaining original waypoints: "

f"{[i + 1 for i in helper_remaining]}"

)

with helper.lock:

helper.preempt_requested = False

success = self.execute_reassigned_task(

helper,

helper.original_area,

helper.original_waypoints,

helper_remaining,

f"{helper.name}: RESUMING ORIGINAL SURVEY"

)

if not success:

print(

f"ERROR: {helper.name} could not "

f"complete its original survey."

)

return False

else:

with helper.lock:

helper.preempt_requested = False

helper.set_state("AVAILABLE")

self.reallocation_done = True

print()

print("=" * 60)

print("REALLOCATION COMPLETE")

print("=" * 60)

print(

f"{helper.name} completed Drone0's "

f"remaining Area A work."

)

if helper.original_area:

print(

f"{helper.name} also completed "

f"its original Area "

f"{helper.original_area} work."

)

return True

# INITIAL TAKEOFF

def takeoff_all(self):

print()

print("=" * 60)

print("TAKING OFF ALL DRONES")

print("=" * 60)

results = {}

threads = []

def takeoff(drone):

try:

print(

f"{drone.name}: ARMING..."

)

drone.arm()

print(

f"{drone.name}: OFFBOARD..."

)

drone.offboard()

print(

f"{drone.name}: TAKING OFF..."

)

result = drone.takeoff(

TAKEOFF_HEIGHT,

TAKEOFF_SPEED

)

results[drone.name] = bool(result)

print(

f"{drone.name}: TAKEOFF "

f"{'SUCCESS' if result else 'FAILED'}"

)

except Exception as exc:

print(

f"{drone.name}: takeoff error: "

f"{exc}"

)

results[drone.name] = False

for drone in self.drones.values():

thread = threading.Thread(

target=takeoff,

args=(drone,),

daemon=True

)

threads.append(thread)

thread.start()

for thread in threads:

thread.join()

if not all(results.values()):

print()

print(

"ERROR: One or more drones "

"failed to take off."

)

return False

print()

print(

"All drones airborne."

)

print(

f"Stabilizing for "

f"{STABILIZATION_TIME:.1f} seconds..."

)

time.sleep(

STABILIZATION_TIME

)

return True

# START INITIAL SURVEYS

def start_initial_surveys(self):

print()

print("=" * 60)

print("STARTING PARALLEL AREA SURVEY")

print("=" * 60)

for drone_name, area_name in INITIAL_ASSIGNMENT.items():

drone = self.drones[drone_name]

waypoints = AREAS[area_name]

delay_enabled = (

drone_name == DELAY_DRONE

)

print(

f"{drone_name} -> Area {area_name}"

)

drone.thread = threading.Thread(

target=drone.run_initial_survey,

args=(

self,

area_name,

waypoints,

delay_enabled

),

daemon=True

)

drone.thread.start()

# WAIT FOR DELAY

def wait_for_delay_detection(self):

print()

print("=" * 60)

print("MISSION MANAGER ACTIVE")

print("=" * 60)

print(

f"Waiting for {DELAY_DRONE} "

f"delay detection..."

)

drone0 = self.drones["drone0"]

while not drone0.delay_event.wait(0.5):

if drone0.get_state() == "FAILED":

print(

"ERROR: Drone0 failed before "

"delay condition."

)

return False

print()

print(

"MISSION MANAGER: DELAY DETECTED"

)

return True

def wait_for_remaining_threads(self):

print()

print("=" * 60)

print("WAITING FOR SURVEY THREADS")

print("=" * 60)

for name, drone in self.drones.items():

if drone.thread is None:

continue

# Helper thread was already joined during

# preemption.

if (

self.reallocation_helper == name

and not drone.thread.is_alive()

):

continue

drone.thread.join()

print(

f"{name}: thread completed "

f"with state "

f"{drone.get_state()}"

)

# FINAL VERIFICATION

def verify_mission(self):

print()

print("=" * 60)

print("FINAL MISSION VERIFICATION")

print("=" * 60)

all_complete = True

for area_name, waypoints in AREAS.items():

completed = self.area_progress(

area_name

)

total = len(waypoints)

print(

f"Area {area_name}: "

f"{completed}/{total}"

)

if completed != total:

all_complete = False

missing = []

for i in range(total):

if (

area_name,

i

) not in self.completed_work:

missing.append(

i + 1

)

print(

f"  Missing waypoints: "

f"{missing}"

)

print()

if self.reallocation_done:

print(

"Dynamic reallocation: COMPLETED"

)

print(

f"Helper drone: "

f"{self.reallocation_helper}"

)

else:

print(

"Dynamic reallocation: "

"NOT REQUIRED"

)

print()

if all_complete:

print("=" * 60)

print("ALL SURVEY AREAS COMPLETED")

print("=" * 60)

print(

"Area A: COMPLETE"

)

print(

"Area B: COMPLETE"

)

print(

"Area C: COMPLETE"

)

print()

print(

"DYNAMIC MULTI-DRONE TASK "

"REALLOCATION: SUCCESS"

)

else:

print("=" * 60)

print("MISSION VERIFICATION FAILED")

print("=" * 60)

return all_complete

# LAND ALL DRONES

def land_all(self):

print()

print("=" * 60)

print("LANDING ALL DRONES")

print("=" * 60)

threads = []

def land(drone):

try:

print(

f"{drone.name}: LANDING..."

)

result = drone.land(

LAND_SPEED

)

print(

f"{drone.name}: LAND "

f"{'SUCCESS' if result else 'FAILED'}"

)

except Exception as exc:

print(

f"{drone.name}: landing error: "

f"{exc}"

)

for drone in self.drones.values():

thread = threading.Thread(

target=land,

args=(drone,),

daemon=True

)

threads.append(thread)

thread.start()

for thread in threads:

thread.join()

print()

print(

"All landing commands completed."

)

# SAFETY STOP

def stop_active_behaviors(self):

for drone in self.drones.values():

try:

if drone.go_to.is_running():

print(

f"{drone.name}: stopping "

f"active GoTo..."

)

drone.go_to.stop()

except Exception:

pass

# MAIN EXECUTION

def execute_mission(self):

# Create drones

print()

print("=" * 60)

print("INITIALIZING DRONE INTERFACES")

print("=" * 60)

for name in [

"drone0",

"drone1",

"drone2"

]:

print(

f"Creating interface: {name}"

)

self.drones[name] = SurveyDrone(

name

)

print()

print(

"All drone interfaces initialized."

)

# Takeoff

if not self.takeoff_all():

return False

# Start parallel survey

self.start_initial_surveys()

# Wait for Drone0 delay

delay_detected = (

self.wait_for_delay_detection()

)

if not delay_detected:

return False

reallocation_success = (

self.perform_reallocation()

)

if not reallocation_success:

print()

print(

"ERROR: Dynamic task "

"reallocation failed."

)

return False

self.wait_for_remaining_threads()

mission_success = (

self.verify_mission()

)

# Land

self.land_all()

return mission_success

def main():

rclpy.init()

manager = AreaSurveyManager()

success = False

try:

success = manager.execute_mission()

except KeyboardInterrupt:

print()

print(

"Keyboard interrupt received."

)

manager.stop_active_behaviors()

except Exception as exc:

print()

print("=" * 60)

print("MISSION EXCEPTION")

print("=" * 60)

print(exc)

manager.stop_active_behaviors()

finally:

print()

print("=" * 60)

print("SHUTTING DOWN DRONE INTERFACES")

print("=" * 60)

for drone in manager.drones.values():

try:

drone.shutdown()

except Exception:

pass

if rclpy.ok():

rclpy.shutdown()

print()

if success:

print("=" * 60)

print("MISSION COMPLETED SUCCESSFULLY")

print("=" * 60)

else:

print("=" * 60)

print("MISSION DID NOT COMPLETE")

print("=" * 60)

if __name__ == "__main__":

main()